AI 201 Project Proposal

Title: Pilot Machine Learning Model for Fuel Requirement Estimation in Agricultural Mechanization

Project Members: Ronald Melvin R. Rosas, Marcus Rafael Tiongson

- Introduction

Fuel consumption is a critical operational component in agricultural mechanization, directly affecting production cost and efficiency. In many contexts, including the Philippines, estimation of fuel requirements is often based on generalized assumptions or static coefficients that fail to account for variability in machine characteristics, operational conditions, and field environments.

Traditional engineering-based models provide approximate estimates but are limited in capturing nonlinear relationships among variables such as horsepower, load, operation type, and field conditions. With the increasing availability of mechanization-related datasets, there is an opportunity to apply machine learning techniques to improve the accuracy and adaptability of fuel consumption estimation.

This study proposes the development of a pilot machine learning model to estimate fuel requirements for selected agricultural machinery operations using structured operational data.

- Problem Statement

Current approaches to estimating fuel consumption in agricultural machinery are:

- Based on static or generalized coefficients

- Not localized to actual operating conditions

- Unable to capture nonlinear interactions among variables

As a result, there is a need for a data-driven model that can:

- Predict fuel consumption more accurately

- Adapt to different machine types and operations

- Provide a basis for improved decision-making in mechanized agriculture

- Objectives

General Objective

To develop and evaluate a machine learning model for estimating fuel requirements of selected agricultural machinery operations.

Specific Objectives

- Identify key variables affecting fuel consumption in agricultural machinery

- Develop a baseline estimation model using statistical methods

- Implement a machine learning model (Random Forest Regressor) for fuel prediction

- Compare the performance of baseline and machine learning models using standard evaluation metrics

- Perform feature importance and model interpretability analysis using SHapley Additive exPlanations (SHAP) and Local Interpretable Model-Agnostic Explanations (LIME)

- Scope and Delimitation

To ensure feasibility within the project timeline, the study is limited to:

- Geographic Scope: Selected region(s) in the Philippines

- Machinery Scope: One to two machinery types (e.g., tractors)

- Operation Scope: Selected operations such as plowing and harrowing

- Data Scope: Secondary data only (no primary field data collection)

The study excludes:

- Real-time sensor data acquisition

- Nationwide modeling

- Full mechanization system simulation

- Deployment of a production system

- Review of Related Literature

  - Fuel Consumption Theory in Agricultural Machinery

Fuel requirement estimation in agricultural mechanization is fundamentally based on the relationship between engine power, implement load, and operational efficiency. Fuel consumption is generally proportional to the power requirement of the machine during operation, making it a practical indicator of energy demand in agricultural systems (Domsch et al., 1999).

However, fuel consumption is not determined solely by engine characteristics. It is influenced by multiple interacting factors, including implement type, soil conditions, machine configuration, and operational mode. These relationships indicate that fuel estimation is inherently a multivariate problem, where several variables simultaneously affect the outcome. While traditional engineering models provide structured computation methods, they rely on simplified assumptions and controlled conditions, limiting their applicability in real-world environments.

  - Empirical Modeling of Fuel Consumption and Field Capacity

Empirical studies provide strong evidence that fuel consumption varies significantly under actual operating conditions. A case analysis conducted by the Philippine Center for Postharvest Development and Mechanization (PHilMech) on commercially available rice combine harvesters demonstrates that fuel consumption is directly related to engine power, efficiency, and operational parameters (PHilMech, n.d.).

The study uses specific volumetric fuel consumption (SVFC) as a standard metric and models fuel consumption as a function of engine power, fuel density, and efficiency. Results indicate that fuel consumption increases with the rated power of the machine, confirming horsepower as a key determinant of fuel use (PHilMech, n.d.).

Furthermore, the study highlights that accurate estimation requires controlling several external variables, including soil condition, crop condition, operator skill, field size, terrain, and environmental conditions. This implies that fuel consumption is not solely inherent to the machine but is significantly affected by operational and environmental variability

In addition, the joint analysis of fuel consumption and field capacity underscores the relationship between energy use and productivity. Fuel consumption is closely linked with operational efficiency, suggesting that estimation models must incorporate both machine performance and field efficiency indicators.

  - Real-World Fuel Consumption Variability

Real-world studies further emphasize the variability of fuel consumption across agricultural systems. A study on diesel-powered irrigation systems reports an average fuel consumption of approximately 74.55 liters per hectare annually (Agaton & Guno, 2024). The study identifies several influencing factors, including soil type, environmental conditions, crop water requirements, distance to water source, and system efficiency.

Although the study focuses on irrigation systems rather than field machinery, it provides important insights into the context-dependent nature of fuel consumption. These findings suggest that fuel use varies significantly across locations and operational conditions, reinforcing the need for models that can adapt to diverse agricultural environments.

  - Agricultural Mechanization and Energy Demand Context

Agricultural mechanization is a key driver of productivity, but it also increases energy demand. Mechanization levels, often measured in horsepower per hectare, reflect the degree of energy input in agricultural production (Bautista et al., 2024).

However, adoption remains constrained due to economic and structural challenges. Fuel and maintenance costs are among the major operational barriers faced by farmers, affecting sustainability and adoption of mechanization technologies (Espino et al., 2024). This highlights the importance of accurate fuel estimation for cost management and planning.

  - Structural and Systemic Constraints in Philippine Agriculture

Beyond technical factors, fuel consumption patterns are also influenced by broader structural and institutional conditions. An industry analysis of Philippine agriculture highlights key constraints such as climate variability, policy fragmentation, and limited access to technology and financing (Ichwandiani & Hassan, 2025).

The agricultural sector operates within a complex system of interdependent stakeholders, including farmers, suppliers, financial institutions, government agencies, and market actors. These relationships determine access to critical resources such as machinery, inputs, credit, and market information, which in turn affect production efficiency and resource utilization

Additionally, inefficiencies in supply chains and the dominance of intermediaries often limit farmer profitability and constrain investment in mechanization. Smallholder farmers, who comprise the majority of agricultural producers, frequently lack access to modern technologies and infrastructure, further widening productivity gaps

These findings imply that fuel consumption is not solely a technical issue but is also shaped by economic and institutional factors. As such, estimation models must consider contextual variability to ensure applicability across different agricultural settings.

  - Synthesis and Research Gap

The reviewed literature establishes that:

- Fuel consumption is a function of engine power, machine characteristics, and operational conditions (Domsch et al., 1999).

- Empirical studies confirm that fuel consumption increases with machine power and varies significantly depending on environmental and operational factors

- Real-world studies highlight the strong influence of site-specific conditions on fuel use (Agaton & Guno, 2024).

- Mechanization increases energy demand and cost sensitivity, making fuel estimation essential for planning and sustainability (Bautista et al., 2024; Espino et al., 2024).

- Structural constraints in the Philippine agricultural sector further influence resource use and mechanization efficiency

Despite these findings, a critical gap remains. Existing approaches to fuel estimation are largely based on deterministic formulas and controlled experimental conditions. These methods often:

- rely on fixed coefficients and assumptions,

- have limited adaptability across varying field conditions, and

- do not fully capture complex interactions among multiple variables.

  - Relevance to the Present Study

This study addresses the identified gap by developing a pilot machine learning model for fuel requirement estimation. The proposed model aims to:

- integrate multiple influencing variables, including machine, operational, and environmental factors;

- capture complex relationships in fuel consumption that are not easily represented by deterministic models; and

- provide scalable and adaptable predictions applicable to real-world agricultural conditions.

By combining engineering-based insights with data-driven modeling approaches, the study contributes to improving the accuracy, efficiency, and applicability of fuel estimation in agricultural mechanization.

- Conceptual Framework

The study is based on the following predictive relationship:

Fuel Consumption = f(Machine Characteristics, Operation Characteristics, Field/Location Conditions, Operational Effort)

The target variable is fuel consumption, measured or estimated in liters. Predictor variables include machine type, horsepower, operation type, area covered, operating time, field or location indicators, and derived efficiency indicators. The baseline model provides a simple reference estimate, while the Random Forest model learns patterns from the data and captures nonlinear interactions among predictors.

- Dataset Description

The dataset will consist of structured records of agricultural machinery operations. Each row will represent one operation instance, such as one machine performing one operation over a defined area and time period.

| Variable Group | Variables | Purpose |
| --- | --- | --- |
| Identification | Record ID, date, region, province, municipality | Traceability, geographic grouping, and data cleaning |
| Machine characteristics | Machine type, horsepower, fuel type, and machine age | Represents equipment-related fuel demand |
| Operation characteristics | Operation type, implement used if available, number of passes if available | Represents the type and intensity of field operation |
| Operational effort | Area covered, operating time, travel distance | Normalizes fuel use per hectare or per hour |
| Field/location indicators | Crop type, soil type, terrain, irrigation/rainfed status if available | Captures contextual variability |
| Target variable | Fuel consumption in liters | Output variable for model training and evaluation |
| Derived variables | Fuel per hour, fuel per hectare, time per hectare | Supports exploratory analysis and model interpretation |

Datasets

- Agricultural Machinery Testing and Evaluation Center (AMTEC)
(https://bit.ly/AMTECTestReports2026)
→ fuel consumption and performance data

- Agricultural and Biosystems Engineering Management Information System (ABEMIS)
(https://abemis.bafe.gov.ph/user/login)
→ machinery inventory and distribution data

- Regional Agricultural Engineering Division (RAED) Cropping Calendar
→ crop stage and seasonal information

- Methodology

  - Research Design

The study will use a comparative predictive modeling design. A baseline model will be developed first, followed by a Random Forest Regressor. The two approaches will be evaluated using the same dataset and regression metrics.

  - Data Preprocessing

- Data consolidation from available secondary sources;

- Data cleaning, including handling missing values and removing inconsistent records;

- Standardization of units, especially hectares, hours, horsepower, and liters;

- Encoding of categorical variables such as machine type and operation type;

- Outlier identification and treatment; and

- Creation of derived variables such as fuel per hectare and fuel per hour.

  - Baseline Model

- Compute average fuel consumption per operation

- Implement simple linear regression

  - Machine Learning Model

The primary model to be implemented is: Random Forest Regressor

Steps:

- Split the dataset into training (70%) and testing (30%)

- Train model on selected features

- Tune parameters (e.g., number of trees, depth)

  - Model Evaluation

Performance will be evaluated using:

- Mean Absolute Error (MAE)

- Root Mean Squared Error (RMSE)

- Coefficient of Determination (R²)

Comparison will be made between:

- Baseline model

- Random Forest model

  - Model Interpretability

Performance will be evaluated using:

- Mean Absolute Error (MAE)

- Root Mean Squared Error (RMSE)

- Coefficient of Determination (R²)

Comparison will be made between:

- Baseline model

- Random Forest model

- Expected Outputs

- Cleaned integrated dataset

- A trained machine learning model for fuel estimation

- Comparative performance analysis of models

- Estimated fuel consumption values for selected operations

- Identification of key variables influencing fuel use

- SHAP feature importance analysis

- LIME local explanations

- Software and Tools

Programming Environment

- Python (Jupyter Notebook)

Libraries

- pandas (data handling)

- numpy (numerical computation)

- scikit-learn (machine learning)

- matplotlib / seaborn (visualization)

- SHAP/LIME (model interpretability)

- Hardware Requirements

- Laptop or desktop computer

- Minimum 8 GB RAM (16 GB recommended)

- Standard CPU (no GPU required)

- Significance of the Study

This study contributes to:

- Demonstrating the applicability of machine learning in agricultural and biosystems engineering problems

- Providing a pilot framework for data-driven fuel estimation

- Establishing a basis for future integration into decision-support systems

- References

Agaton, C. B., & Guno, C. S. (2024). Promoting sustainable agriculture using solar irrigation: Case study of small-scale farmers in the Philippines.

Bautista, C. J., Quipo, J., & Taldhay, M. (2024). Enhancing agricultural productivity in the Philippines: A comprehensive review of mechanization status and sustainable strategies. European Journal of Science, Innovation and Technology.

Briones, R. M. (2021). Philippine agriculture: Current state, challenges, and ways forward. Philippine Institute for Development Studies.

Domsch, H., Ehlert, D., Smrikarov, A. S., & Bentscheva, N. V. (1999). Fuel consumption measurement with agricultural machinery. Landtechnik, 54(5), 278–279.

Espino, L. C., Lutarte, R. J., Traya, A. J. G., & Antipolo, A. G. (2024). Agricultural mechanization policy: Farmers in focus. International Journal of Research and Innovation in Social Science.

Ichwandiani, R., & Hassan, S. H. (2025). Industry analysis: Agriculture in the Philippines. ASEAN Research Center.

Philippine Center for Postharvest Development and Mechanization (PHilMech). (n.d.). Analysis of the technical performance of commercially available rice combine harvesters.