Y I

T

S

OF THE

P

H

I

L

CHINERY TESTING AND EVA

R

NU VI E

190 8

1977

PI P NI ES

A

M

L

A

R

U

GA R CI ULT

L

U

T

A

OI

N

CENT RE

TEST REPORT NO. 2013 - 142

CLAAS Corn Combine Harvester (B)

Model: Crop Tiger 30 Tiera Tract

College of Engineering and Agro-Industrial Technology University of the Philippines Los Baños,

College, Laguna, 4031 Philippines

Tel/Fax No. (049) 536-2527 e-mail: amtec@uplb.edu.ph http.//amtec.uplb.edu.ph

CLAAS CROP TIGER 30 TERRA TRAC MULTI-CROP COMBINE HARVESTER (tested for Corn) - B

Test Report No. : 2013 - 142

Date of Test : March 13, 2013

Test Valid Until : March 13, 2018

Brand : CLAAS

Model : CROP-TIGER 30 TERRA TRACT Type : Ride-on, crawler-type, tank-type Manufacturer : CLAAS INDIA PRIVATE, LTD.

Test Requested By : AGRI COMPONENT CORPORATION National Highway, Tagaran

Cauayan City, Isabela

Machine Classification : Commercial

SUMMARY

The CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester was  tested using Dekalb 9132 corn variety with a Grain Moisture Content of 25.6%, wet  basis, Corn Kernel-Corn Ear Ratio of 0.80 and average Plant Height of 2540 mm.  This was the second test for the same corn combine harvester requested by the same  company. The first test was conducted on September 8, 2012 with Test Report No.  2012-246.

Based on the results of the test, the CLAAS Crop Tiger 30 Terra Trac Multi Crop Combine Harvester had an Actual Field Capacity of 0.391 ha/h or 2.56 h/ha and  Field Efficiency of 77.6% at an average Travelling Speed of 2.84 kph. It had also a Shelled Grain Output Capacity of 3861 kg/h with shelled grain Purity of 98.26%. The  corn combine harvester had a Total Grain Loss of 9.33% consisting of 8.86%  Unpicked Loss, 0.02% Blower Loss, 0.30% Separation Loss, 0.15% Unshelled Loss  and no Uncut Loss.

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

2

for Corn) - B……

Grain quality analysis of threshed grains showed a net percent Cracked Grain  of 5.33 % and Broken Grains of 1.94%

The Fuel Consumption Rate of the corn combine’s 44.2 kW diesel engine was  5.77 L/h.

During the test, three persons were needed to operate the corn combine: one  for operating the machine and two for bagging.

No breakdown or failure of components occurred during the test.

PURPOSE AND SCOPE OF TEST

The machine was tested for verification of its performance. The test was  conducted at a farmer’s field in Brgy. Villa Luna, Cauayan City, on March 13, 2013 and was evaluated on the following items:

1. Actual Field Capacity

2. Field Efficiency

3. Output Capacity

4. Grain losses

a. Unpicked Loss

b. Separation Loss

c. Blower Loss

d. Uncut Loss

e. Unshelled Loss

5. Fuel Consumption

OPERATOR’S MANUAL

The operator’s manual submitted with the machine contains the following:  Key Points of Safe Operation; Safety Signs and Positions to be Posted; Position and  Functions of Each Unit; Check Before Operation; Travel and Transportation; Method  of Harvesting; Maintenance; General Faults and Solutions; Technical Specifications;  and, After Sales Services.

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

3

for Corn) - B……

DESCRIPTION OF THE MACHINE

The CLAAS Crop-Tiger 30 Terra Trac Multi-Crop Combine Harvester (Figure 1), was a ride-on, crawler-type, tank-type harvesting machine which  combined the cutting and shelling operations in one machine. It was powered by a  diesel engine which drove the transmission systems of the machine. The corn combine consisted of reel assembly, cutting assembly, screw and chain conveyors, shelling assembly, traveling device and control levers.

The reel assembly (Figure 2) with its rotary motion guides the corn plant to  the cutting assembly. It consisted of five anchor bars.

The machine’s cutting device consisted of reciprocating cutting knives (Figure 3). It had a cutting width of 2.0 m and the height of cut was adjustable. The corn combine’s delivery device to its shelling unit consisted of a screw  conveyor (Figure 4a) and a chain conveyor (Figure 4b). The screw conveyor  conveyed the cut plants to a chain conveyor which in turn delivered the cut plants to  the shelling chamber of the combine.

The shelling assembly was an axial-flow type and consisted of an open-type  cylinder.

The cleaning assembly consisted of a centrifugal fan which separated the  small corn cobs and other light impurities from the shelled grain and a double  layer oscillating screen.

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

4

for Corn) - B……

The corn combine was provided with a grain tank (Figure 5) which held the  shelled grain and a folding-type discharge chute (Figure 6) with screw conveyor  where shelled grains were discharged.

The corn combine harvester was provided with control levers and control  panel (Figure 7). The control levers were the following: accelerator lever; shelling clutch lever, main shift lever; reel lifting lever; and, steering and cutting table  operating lever. The control panel houses the speedometer and pilot lights for water  temperature, oil pressure, and battery charge.

The machine had a crawler-type (Figure 8) traveling device.

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

5

for Corn) - B……

Figure 1. CLAAS Multi-Crop Combine Harvester

Model: Crop-Tiger 30 Terra Trac

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

6

for Corn) - B……

Figure 2. Reel Assembly

Figure 3. Reciprocating cutting knife

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

7

for Corn) - B……

Figure 4. a) Screw conveyor

b) Chain conveyor

Figure 5. Grain Tank

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

8

for Corn) - B……

Figure 6. Folding-type discharge chute with

screw conveyor

Figure 7. Control levers and control panel

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

9

for Corn) - B……

Figure 8. Crawler-type traveling device

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

10

for Corn) - B……

RESULTS AND DISCUSSIONS

Results of field performance tests are shown in Table 2.

Performance

The performance of the corn combine was evaluated in terms of its field capacity, field efficiency, output capacity, grain losses and grain quality.  Actual Field Capacity. It is the ratio of the area harvested and the total time of  operation. The Actual Field Capacity of the machine was 0.391 ha/h or 2.56 h/ha. Field Efficiency. It is the ratio of the Actual Field Capacity and the Theoretical  Field Capacity, expressed in percent. Based on the Theoretical Field Capacity of 0.504 ha/h, the Field Efficiency of the machine was 77.6%.

Unpicked Loss. This refers to the kernels of the corn ears that have fallen to  the ground during the machine’s cutting operation. Unpicked Loss was 8.86%. Separation Loss. This refers to the grains that come out of the shelling chamber with the cut plants. Separation Loss was 0.30%.

Blower Loss. This refers to the grains that come out of from the blower outlet.  Blower Loss was 0.02%.

Uncut Loss. This refers to the kernels of the corn ears from the corn plant that  were not cut or missed by the reaping mechanism of the machine. There was no Uncut Loss.

Unshelled Loss. This refers to grains that remained on the corn ears fed into  the shelling chamber. Unshelled Loss was 0.15%.

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

11

for Corn) - B……

Grain quality. Quality of shelled grain was determined in terms of percent  cracked and broken grains. Results showed that the shelled output had a net  percentage Cracked Grains of 5.33 % and Broken Grains of 1.94 %. (Cracked  grains Refer to those grains whose endosperms are partially fractured while broken  grains refer to those grains that are split into 2 or more pieces.)

Fuel Consumption

The corn combine’s diesel engine had a Fuel Consumption Rate of 5.77 L/h.

Labor Requirement

Three persons were needed during operation of the corn combine (Figure 9):  one for operating the machine and two for bagging.

Observations

1. The corn combine was provided with an operator seat (Figure 10a) and a  canopy (Figure 10b) for the convenience of the operator.

2. The corn combine was easy to maneuver at corners (Figure 11). 3. When the grain tank was full, shelled grains were discharged through the  folding-type discharge chute (Figure 12) with screw conveyor.

4. The corn combine was provided with stickers (Figure 13) on safety.

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

12

for Corn) - B……

a) Machine operator

b) Baggers

Figure 9. Labor requirements during machine operation

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

13

for Corn) - B……

Figure 10. a) Operator’s seat

b) Canopy

Figure 11. Turning at corner

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

14

for Corn) - B……

Figure 12. Discharge of shelled grains

Figure 13. Sticker attached to the corn combine

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

15

for Corn) - B……

FIELD PERFORMANCE TEST

Machine tested : CLAAS Multi-Crop Combine Harvester

(tested for Corn) - B

Model : Crop Tiger 30 Terra Tract

Location of test: Brgy. Villa Luna

Cauayan City

Date of test : March 13, 2012

Table 2. Results of field performance test

ITEMS TEST DATA*

A. Condition of Crop

1. Variety : DEKALB 9132

2. Number of Plants/Hill : 3

3. RowSpacing (mm) : 700

4. Plant Height (mm) : 2540

5. Corn Ear Dimension, L x D (mm) : 145 x 41

6. Corn Kernel – Corn Ear Ratio : 0.80

7. Moisture Content of Grains (%) : 25.6

B. Field Conditions

1. Type of Soil : Sandy loam

2. Soil Resistance (kg/cm2) : 6.67

* Average of two test trials

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

16

for Corn) - B……

Table 2. Results of field performance test (cont’n.)

ITEMS TEST DATA*

C. Field Performance

1. Method of Operation : Alternating Pattern 2. Travelling Speed (kph) : 2.84

3. Effective Cutting Width (mm) : 2100

4. Height of Cut (mm) : 660

5. Actual Field Capacity

a) ha/h : 0.391

b) h/ha : 2.56

6. Theoretical Field Capacity

a) ha/h : 0.504

b) h/ha : 1.98

7. Field efficiency (%) : 77.6

8. Shelled Grain Output

Capacity (kg/h) : 3861

9. Shelled grain unloading rate (kg/h) : 39808

10. Purity of shelled grain (%) : 98.26  11. Unpicked Loss (%) : 8.86

12. Blower Loss (%) : 0.02

13. Separation Loss (%) : 0.30

* Average of two test trials

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

17

for Corn) - B……

Table 2. Results of field performance test (cont’n.)

ITEMS TEST DATA*

14. Unshelled Loss (%) : 0.15

15. Uncut Loss (%) : Nil

16. Total Grain Loss (%) : 9.33

17. Noise level, db(A) : No Data

18. Fuel consumption, (L/h) : 5.77

D. Laboratory Grain Analysis

1. Before Harvest

a) Cracked (%) : 0

b) Broken (%) : 0

2. After Harvest

a) Cracked (%) : 5.33

b) Broken (%) : 1.94

3. Net Percent

a) Cracked : 5.33

b) Broken : 1.94

* Average of two test trials

CLAAS Crop Tiger 30 Terra Trac Multi-Crop Combine Harvester (tested

18

for Corn) - B……

Test Engineer:

________________________

DARWIN C. ARANGUREN

Checked by:

____________________

ROMULO E. EUSEBIO

Approved for release:

___________________________

DELFIN C. SUMINISTRADO

Director

_______________________

Date

N.B.

This test report should not be used for advertisement or for promotion of the  product but only for evaluation purposes and should not be reproduced in whole or in  part without permission of AMTEC.