Testing

and Ev

OF THE OS BANOS

BOS

chinery

INIVERSITY

AMTECO

Wonenie

PHILIPPINE

ultural

OPINES

CEAT

by 19

Center

577

AGRICULTURAL MACHINERY TESTING AND EVALUATION CENTER

LOS BANOS

Test Report No. 2017-0081

Agricultural Machinery - Centrifugal Pump

SAKAI NS-100 Packing Type

College of Engineering and Agro-Industrial Technology, University of the Philippines Los Baños, College, Laguna 4031 Email: amtec.uplb@up.edu.ph Facebook: www.facebook.com/amtec.uplb Website: www.amtec.uplb.edu.ph

Contact numbers: (+63) 921 400 7137 or (+63) 977 204 7236

TR No. 2017-0081 (SAKAI NS-100 Packing Type)

Page 2 of 10

2017 - 0081

Test Report Number

Date of Issuance

March 13, 2019 March 13, 2024

Valid Until

Classification

Commercial

Agricultural Machine

Pump

Type

Single-end Suction, 100 x 100 mm, Self-Priming Centrifugal pump SAKAI

Brand

Model

NS-100 Packing Type

Machine Serial Number

No serial number

Manufacturer

YANGCHUN MUNICIPAL DAFENG MACHINERY & EQUIPMENT CO., LTD., Guangdong, China ALL CERTIFIED EQUIPMENT TRADING CORP. 905 EDSA Philam, Quezon City

Test Requested by

None

Previous Test Reports of similar Brand and Model

SUMMARY

The SAKAI NS-100 Packing Type Self-Priming Centrifugal Pump was tested for performance. It was tested at the recommended pump shaft speed of 2000 rpm.

The Maximum Total Head (H) was 22.6 m which occurred at Zero Discharge

condition. The Maximum Discharge Capacity of 30.5 L/s occurred at 5.28 m Total Head and at Input Power to the Pump shaft of 7.78 kW.

The Maximum Pump Efficiency of 47.8 % was attained at a Discharge Rate of 17.1 L/s. At this optimum discharge condition, the Total Head was 17.7 m and the Input Power to the Pump shaft was 6.23 kW.

The suction condition of the pump was also tested. With zero Discharge, the Maximum Total Suction Lift was 5.74 m. The Net Positive Suction Head Available at this

point was 4.22 m.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0081 (SAKAI NS-100 Packing Type)

Page 3 of 10

Table 1 compares the test results of the SAKAI NS-100 Packing Type Self-Priming Centrifugal Pump in comparison with its rated performance as claimed by the manufacturer.

Table 1. Performance of the SAKAI NS-100 Packing Type Centrifugal Pump

Performance Criteria

Rated Performance

AMTEC Test Maximum Discharge Capacity, L/s

28.4

30.5 Maximum Total Head, m

25.3

22.6 Rated Power, kW

7.46

7.78* Rated Speed, rpm

2000

2000 *Input Power at Maximum Discharge Capacity

PURPOSE AND SCOPE OF TEST

The purpose of the test was to establish the pump performance at the recommended pump shaft speed which included the Total Head, Input Power and Pump Efficiency at varying discharge capacities and suction conditions. The pump was mounted on the

laboratory test set-up shown in Figure 1.

The test was conducted on October 10, 2017 at the AMTEC Test Laboratory, UPLB,

College, Laguna.

Figure 1. SAKAI NS-100 Packing Type Self-Priming

Centrifugal Pump on the test set-up

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0081 (SAKAI NS-100 Packing Type)

Page 4 of 10

METHOD OF TEST

The pump was tested following the “Philippine Agricultural Engineering Standards, PAES 115:2000, Agricultural Machinery – Centrifugal, Mixed-Flow and Axial-Flow Water Pumps - Methods of Test”.

DESCRIPTION OF THE PUMP

The SAKAI NS-100 Packing Type Self-Priming Centrifugal Pump (Figure 2) was a single-end suction, 100 x 100 mm, Centrifugal pump. The suction inlet and discharge outlet were threaded and designed for a 100 mm diameter pipe. The impeller was a semi-open type made of cast iron material connected to the pump shaft through a thread with locknut. It was housed in a cast iron volute casing with priming chamber provided with priming inlet and drain outlet. The discharge outlet was located on top of the pump facing upward, while the suction inlet was placed near the top portion of the chamber. A rubber swing type one-way valve was placed at the inlet to maintain sufficient amount of liquid inside the chamber necessary for priming. The stuffing box used a packing seal. The pump shaft rotated on ball bearing housing bolted to the casing. Table 2 shows the detailed specifications of the pump.

Figure 2. SAKAI NS-100 Packing Type Self-Priming Centrifugal Pump

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0081 (SAKAI NS-100 Packing Type)

Page 5 of 10

PUMP SPECIFICATIONS

Table 2. Detailed specifications of the SAKAI NS-100 Packing Type Centrifugal Pump

Brand

SAKAI

Model

NS-100 Packing Type

Serial No.

No serial number

Make

YANGCHUN MUNICIPAL DAFENG MACHINERY & EQUIPMENT CO., LTD., Guangdong, China Single-end Suction, Centrifugal Pump 100 x 100

5.

Type

457

248

405 35.0

Size (mm) Overall Dimension and Weight 7.1 Length (mm) 7.2 Width (mm) 7.3 Height (mm) 7.4 Weight (kg) Impeller 8.1 Type 8.2 Diameter (mm) 8.3 Width (mm) 8.4 No. of vanes 8.5 Material

Semi-open 200

41.5

Cast iron

Threaded

Suction Inlet 9.1 Type 9.2 Diameter (mm) 9.3 Material

100

Cast Iron

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0081 (SAKAI NS-100 Packing Type

Page 6 of 10

Table 2. Detailed specifications of the SAKAI NS-100 Packing Type Centrifugal Pump

(Continuation)

10.

Discharge Outlet 10.1 Type 10.2 Diameter (mm)

Threaded

100

10.3

Material

Cast Iron

11.

Casing 11.1 Type

Volute

11.2

Material

Cast Iron

12 13.

Stuffing box Rotation

Packing Type Counter Clockwise when Facing the nameplate

14.

2000

15.

Rated speed (rpm) Rated Maximum Discharge Capacity (L/s) Rated Maximum Total Head (m)

28.4

6.

25.3

Ka

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0081 (SAKAI NS-100 Packing Type)

Page 7 of 10

PERFORMANCE TEST

Results of performance test are shown in Table 3 and plotted in Figure 2.

Pump Tested

Model

SAKAI NS-100 Packing Type October 10, 2017

Date of Test

Test Condition

Ambient Temperature (°C) Atmospheric Pressure (mb) Pumping Water Temperature (°C)

27.0 1015 30.0

:

Table 3. Results of Performance Test at 2000 rpm

SHAFT TORQUE (kg-m)

PUMP INPUT POWER (kW)

TOTAL HEAD (m)

DISCHARGE CAPACITY

(L/S)

WATER POWER (kW)

PUMP EFFICIENCY

(%)

1.95

3.99

22.6

2.42

4.97

20.9

8.08

1.66

33.3

2.74

5.63

19.7

12.6

2.44

43.4

3.03

6.23

17.7

17.1

2.98

47.8

3.21

6.58

16.1

19.8

3.12

47.3

3.40

6.97

14.4

22.6

3.19

45.7

3.59

7.36

12.2

25.2

3.02

41.0

3.73

7.66

9.75

27.5

2.63

34.3

3.83

7.86

7.79

29.4

2.25

28.6

3.79

7.78

5.28

30.5

1.58

20.3

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0081 (SAKAI NS-100 Packing Type)

Page 8 of 10

25.0

20.0

Pump Total Head (m), Pump Input Power (kW)

Pump Efficiency (%)

Pump Total Head Pump Input Power

-

Pump Efficiency

TO

0.0

0.0

5.0 10.0 15.0 20.0 25.0 30.0

Discharge Capacity (L/s) Figure 2. Performance Characteristic Curves of the

SAKAI NS100 Packing Type Self-Priming Centrifugal Pump at 2000 rpm shaft speed

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0081 (SAKAI NS-100 Packing Type)

Page 9 of 10

CAVITATION TEST

Results of Cavitation test at 2000 rpm are shown in Table 4.

Pumpset Tested

SAKAI

Model

NS-100 Packing Type October 10, 2017

Date of Test

Test Condition

21.7

Ambient Temperature (°C) Atmospheric Pressure (mb) Pumping Water Temperature (°C)

1015

30.0

Table 4. Results of Cavitation Test at 2000 rpm

TOTAL HEAD

(m)

TOTAL SUCTION | NET POSITIVE

LIFT

SUCTION HEAD AVAILABLE

(m)

DISCHARGE CAPACITY

(L/s)

1.63

8.33

5.28

30.5

2.84

7.12

6.25

27.7

4.41

5.55

7.41

23.1

5.74

4.22

4.22

7.92

0.00

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0081 (SAKAI NS-100 Packing Type)

Page 10 of 10

Prepared by:

CONRAD IAN S. TAN

Test Engineer

Verified by:

DARWIN C. ARANGUREN

Test Engineer

Approved for Release:

ARTHUR L. FAJARDO

AMTEC Director

MAR 13 2019

Date

REMINDER

This test report should not be used for advertisement or for promotion of the product but only for evaluation and verification purposes and should not be reproduced in whole or in part without the permission of AMTEC.

For inquiries regarding the test report, you can visit us at our office, contact us at (+63) 921 400 7137 or (+63) 977 204 7236, send us a message at amtec.uplb@up.edu.ph or visit our official website at amtec.uplb.edu.ph.

Sa

me

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.