Testing

gand Eu

OF THE

BANOS

OS 84

Aachiner

O AMTECO

NIVERSITY

valuation

PHILIPPIN,

ultural M

Thouby

CEAT

Center

1977

AGRICULTURAL MACHINERY TESTING AND EVALUATION CENTER

LOS BANOS

TEST REPORT No. 2017-0244

Agricultural Machinery – Small Engine

CAMEO CP80

College of Engineering and Agro-Industrial Technology, University of the Philippines Los Baños, College, Laguna 4031 Email: amtec.uplb@up.edu.ph Facebook: www.facebook.com/amtec.uplb Website: www.amtec.uplb.edu.ph

Tel/Fax: (049) 536-2527

TR No. 2017-0244 (CAMEO CP80)

Page 2 of 10

Test Report Number

2017-0244

Date of Issuance

December 4, 2017

Valid Until

December 4, 2022

Agricultural Machinery

Small Engine

Type

Four stroke, water-cooled, horizontal single cylinder diesel engine

Brand

CAMEO

Model

CP80

Test Requested by

SIM BENG KO MARKETING CORP. 901 EDSA, Philam Homes, Quezon City

SUMMARY

Performance Criteria

Standard

AMTEC TEST

80

Maximum Power as Percentage of Rated

95.31 Maximum Power, %

Continuous Power as Percentage of Rated

80

91.52 Maximum Power, % Maximum Noise Level (Continuous Running

92* Test), dB(A) 1 Occurred at 2405 rpm, 2.27 kg-m, and 3.48 L/h fuel consumption during varying load test. Power developed at that point was 5.61 kW. 2. Average output power during the five-hour test was 5.38 kW at shaft speed of 2420 rpm and torque of 2.17 kg-m. 3 Allowable noise level for six (6) hours of continuous exposure based on Occupational Safety and Health Standards, Ministry of Labor, Philippines, 1983

86.93

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0244 (CAMEO CP80)

Page 3 of 10

OBSERVATIONS

1. Manufacturer

CHONGQING CAMEL POWER

MACHINERY CO., LTD.

Beibei, Chongqing, China Provided. Presented in English

Yes

Yes

Yes

Yes

Yes

Yes

2. Operator's Manual

a. Safety and Warnings b. Operating Information C. Accessories and Attachments d. Maintenance Instruction e. Storage f. Handling, reception, transportation,

assembly and installation g. Specifications h. Dismantling and disposal i. Warranty

j. Parts list 3. Previous Valid Test Reports of Similar

Brand and Model 4. Ease of starting the engine 5. During Varying Speed Test 6. During Continuous Running Test

Yes

None

None

None

None

Easy (see starting test result)

No breakdown occurred No breakdown occurred

PURPOSE AND SCOPE OF TEST

Purpose Date/s of Test Location of Test

Performance Testing / Commercial July 27, 2017 AMTEC Test Lab, College, Los Baños, Laguna

Items Tested

Operator's Manual, Starting Test, Varying Speed Test, and Continuous Running Test

METHOD OF TEST

The engine was tested based on the Philippine Agricultural Engineering Standards, PAES 117:2000 Agricultural Machinery - Small Engines - Method of Test.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0244 (CAMEO CP80)

Page 4 of 10

DESCRIPTION OF THE ENGINE

Figure 1. CAMEO CP80 Diesel Engine

SPECIFICATIONS

Table 1. Engine Specifications

ITEM

MANUFACTURER'S SPECIFICATIONS

AMTEC VERIFICATION

1.1

1.2

ND ND ND

780 430 460

1.3

| 1.4

ND

841

2 2.1 2.2 2.3 2.4 2.5 2.5.1

Dimensions and weight of engine Overall length, mm Overall width, mm Overall height, mm Weight of the engine, kg Engine specification Brand Model

Serial Number Make Type Based on number of strokes to complete one cycle Based on number of cylinder Based on cylinder arrangement Maximum brake power at rpm, kW

CAMEO

CP80 90-1612256

China

CAMEO

CP80 90-1612256

ND

Four-stroke

Four-stroke

2.5.2 2.5.3 2.6

Horizontal 5.88 kW at 2400 rpm

Horizontal 5.61 kW at 2405 rpm?

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0244 (CAMEO CP80)

Page 5 of 10

5.38 kW at 2400 rpm

NM NM

Diesel Gravity

NM

2.7 Continuous brake power at rpm, 5.15 kW at 2400 rpm

kW 2.8 Bore x stroke, mm

84 x 84 2.9 Displacement volume, cm3

465 2.10 Fuel system 2.10.1 Fuel used

Diesel 2.10.2 Type of fuel feed

ND 2.10.3 Tank capacity, L

10 2.11 Cooling system

Water-cooled 2.12 Lubrication system

Trochoid Pump 2.13 Starting system

Hand cranked 2.14 Governor system 2.15 Air cleaner

Wet type (Oil-bath type) 2.16 Exhaust system

Muffler 2.17 Other attachments/accessories

ND 1 Weight includes fuel, water, and oil. 2. Maximum power generated during varying load test. 3. Set at Continuous power specified by the manufacturer.

ND

Water-cooled

Force-feed Hand cranked

NM Oil-Bath type

Muffler Engine head light

PERFORMANCE TEST

1. STARTING TEST

Table 2. Results of Starting Test

ITEM

COLD STARTING

HOT STARTING

28.0

28.5

Test Condition

- Ambient Temperature, °C

- Relative Humidity, % Fuel Temperature, °C

| Engine Oil Temperature, °C

66.0

65.0

| 2

NM

NM

3

29.0

91.0

4

Number of attempts to start the engine

l

5

Easy of cranking /starting

Easy

Easy

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0244 (CAMEO CP80)

Page 6 of 10

PERFORMANCE TEST

2. VARYING LOAD TEST

Engine on test

CAMEO Diesel Engine

Test Conditions

Model

CP80

Ambient Temperature, °C

28.4.

Date of Test

July 27, 2017

Relative humidity, %

68.0.

Table 3. Results of varying load performance test at 95.3% of the Rated Maximum Power

SPECIFIC ENGINE

TEMPERATURE (°C) SHAFT OUTPUT

FUEL

FUEL NOISE SHAFT

TORQUE

CONSUMP POWER

CONSUMP- LEVEL SPEED

TION

ENGINE EXHAUST COOLING (kg-m)

(kW)

TION (L/h)

[dB(A)] (rpm)

| OIL

AIR WATER

(g/kW-h) 2601 0.75 2.00 1.42 594.2 88.4 93.0 453.0 75.0

2507

1.48

3.82

1.90

417.9

87.4.

95.0

557.0

78.0

2405

2.27

5.61

3.48

521.7

87.2

| 101.0

778.0

97.0

2322

2.34

5.57

3.66

552.3

87.0

105.0

756.0

102.0

2203

2.37

5.36

3.80

595.0

86.6

|

106.0

740.0

99.0

2100

2.36

5.10

3.77

620.7

85.4

108.0

722.0

104.0

2009

2.31

4.77

3.70

651.5

86.0

108.0

706.0

105.0

1910

2.32

4.54

3.66

676.6

84.9

110.0

680.0

106.0

1800

2.41

4.45

3.57

673.5

84.6

110.0

668.0

116.0

1712

2.43

4.27

3.47

682.2

83.8

109.0

657.0

119.0

1611

2.42

4.00

3.40

713.9

82.7

109.0

121.0

1395

2.26

3.24

3.85

998.0

81.4

109.0

588.0

122.0

1214

2.14

2.67

4.16

1308.6

80.6

108.

01

598.0

121.0

OBSERVATIONS/REMARKS: No breakdown occurred.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0244 (CAMEO CP80)

Page 7 of 10

1500.0

1250.0

1000.0

Output Power (kW), Engine Shaft Torque (kg-m), Fuel Consumption (L/h)

750.0

Specific Fuel Consumption (g/kW-h)

500.0

Output Power

Shaft Torque * Fuel Consumption

Specific Fuel Consumption

250.0

0.0 +

1200

0.0 2700

1500

1800

2100

2400

Engine Shaft Speed (rpm)

Figure 1. Performance characteristic curves of the

CAMEO CP80 Diesel Engine

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0244 (CAMEO CP80)

Page 8 of 10

PERFORMANCE TEST

3. CONTINUOUS RUNNING TEST

Engine on test

CAMEO Diesel Engine

Test Conditions

Model

Ambient Temperature, °C

30.5

CP80

57.3

Relative Humidity, % Duration of Test (h)

Date of Test

July 27, 2017

Table 4. Results of continuous running test for five hours at 91.5% of the Rated Maximum

Power

SPECIFIC ENGINE

TEMPERATURE (°C) SHAFT

FUEL OUTPUT

FUEL NOISE TIME SHAFT

TORQUE POWER CONSUMP

CONSUMP- LEVEL (h) SPEED

TION

ENGINE EXHAUST COOLING (kg-m) (kW)

TION

(L/h) (rpm)

[dB(A)]

OIL AIR WATER

(g/kW-h) 2416 2.16 5.37 2.37 370.7 86.8 109 586 94

2418

2.17

5.38

2.35

367.5

85.9 1108

587

93

2424

2.16

5.37

2.35

367.8

86.6

109

588

90

1.5

2419

2.16

5.36

2.34

366.0

86.9

110

593

93

2.0

2417

2.16

5.35

2.32

363.4

87.0

112

596

95

2.5

2418

2.17

5.39

2.37

369.5

87.1

112

598

95

3.0

|

2421

2.17

5.40

2.39

371.0

87.2

112

602

94

3.5

2420

2.17

5.38

2.37

370.3

87.0

112

606

95

4.0

2423

2.17

5.41

2.38

370.4

87.3

111

608

95

4.5

2420

2.16

5.38

2.40

374.6

86.

9

112

606

96

5.0

2421

2.17

5.39

2.39

371.5

87.3

112

608

96

Ave

2420

2.17

5.38

2.37

369.3

86.9

110.8

598.0

94.2

OBSERVATIONS/REMARKS: Engine was set at the continuous power setting specified by the manufacturer. No breakdown occurred.

-

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0244 (CAMEO CP80)

Page 9 of 10

Output Power (kW), Fuel Consumption (L/h)

Output Power

Fuel Consumption + Engine Oil Temperature -- Exhaust Air Temperature

Cooling Water Temperature

Temperature (°C)

0.0 +

0.0

1.0

2.0

3.0

H

4.0

5.0

Time (h)

Figure 2. Performance characteristic curves during the continuous

running of the CAMEO CP80 Diesel Engine

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2017-0244 (CAMEO CP80)

Page 10 of 10

Prepared by:

JASPER CA LANGAN

Test Engineer

Verified by:

noun ROMULO E. EUSEBIO

Test Engineer

Abang DARWIN C. ARANGUREN

Test Engineer

Approved for Release:

JAN 29 2019

Taib ARTHUR L. FAJARDO

AMTEC Director

Date

REMINDER

This test report should not be used for advertisement or for promotion of the product but only for evaluation and verification purposes and should not be reproduced in whole or in part without the permission of AMTEC.

For inquiries regarding the test report, you can visit us at our office, contact us at

09772047236, send us a message at amtec.uplb@up.edu.ph or visit our official website at

amtec.uplb.edu.ph.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-industrial Technology (CEAT) University of the Philippines Los Baños (UPLB)

Falsification of document is punishable by law under Article 172 of the Revised Penal Code.