achinery

cultural/

Agricul

Testing

and

1977

Center

AMTEC

AGRICULTURAL MACHINERY TESTING AND EVALUATION CENTER

UNIVERSITY

OF THE

105

BANOS

CEAT

Test Report No. 2018-0126

PACT

DOC

Agricultural Machinery - Biomass Shredder ACT STANDARD

PHILIPPINES

UNIVER

06500

OF THE

1908

LOS BANOS

College of Engineering and Agro-Industrial Technology, University of the Philippines Los Baños, College, Laguna 4031 Email: amtec.uplb@up.edu.ph Facebook: www.facebook.com/amtec.uplbWebsite: www.amtec.uplb.edu.ph Tel: +63 977 2047 236 / +63 921 4007 137

TR No. 2018-0126 (ACT STANDARD)

Test Report Number

2018-0126

Page 2 of 7

Date of Issuance

Valid Until

Agricultural Machinery

Type

Brand

February 22, 2019

February 21, 2024

Biomass Shredder

Trailer type, horizontal blade assembly, closed

cylinder ACT

Model

Test Requested by

STANDARD

Manufacturer

SUMMARY

Criteria

Availability of outlet chute

Input Capacity, kg/h

As per PNS/PAES 244:2010 Provided Not specified

AMTEC Test

Provided

1541

Noise Level, [db(A)], maximum

96

94.02

OBSERVATIONS

1. Manufacturer

2. Prime Mover

3. Operator's Manual

4. Previous Valid Test Reports of

Similar Brand and Model

5. Operation

a. Loading

ACT MACHINERIES & METALCRAFT CORP., Turayong, Cauayan City, Isabela

5.22 kW YANMAR TF70L Diesel Engine

None

None

b. Collecting output

6. Adjustment

7. Repair of parts

8. Cleaning of parts

Operator was observed to be standing on the shredding cylinder to facilitate loading. Output was thrown into a pile to be bagged after operation.

Not observed

Not observed

Not observed

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2018-0126 (ACT STANDARD)

9. Safety

10. Availability of the switches

needed

11. Ease of transporting the

machine

12. Vibration

13. Number of operator/s

14. Others

Provided with belt guard

No switch

Page 3 of 7

Provided with wheels and drawbar for hitching

Not observed

Two

Part of fed test material are returned occasionally from the hopper.

PURPOSE AND SCOPE OF TEST

Purpose

Date of Test

Location of Test

Items Tested

Performance Testing / Commercial

August 3, 2018

ACT MACHINERIES & METALCRAFT

CORPORATION,

Turayong, Cauayan City, Isabela

Input Capacity; Fuel consumption; Labor requirements; and Noise level

METHODS OF TEST

The machine was tested based on the "PNS/PAES 245:2010 Agricultural Machinery -

Biomass Shredder - Methods of Test."

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2018-0126 (ACT STANDARD)

DESCRIPTION OF THE MACHINE

b

KEF

b

C

a

Page 4 of 7

Figure 1. Parts of the ACT STANDARD powered by YANMAR TF70L Diesel Engine:

a) Intake hopper, b) Shredding cylinder, c) Output chute.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2018-0126 (ACT STANDARD)

SPECIFICATIONS OF THE MACHINE

Page 5 of 7

Manufacturer's

Item

Specification

Verification by the Testing Agency

1

Main Structure

1.1

Overall dimensions, mm

1.1.1

Length

ND

1820

1.1.2

Width

ND

1250

1.1.3

Height

ND

1440

1.2

Weight without primemover

ND

NM

(kg), if applicable

2

Loading hopper

2.1

Material

ND

ND

2.2

Bottom opening, mm

2.2.1

Length

ND

365

2.2.2

Width

ND

250

2.3

Height from the ground, mm

ND

1440

3

Shredder assembly

3.1

Blade cover

3.1.1

Dimensions, L x W, mm

ND

635 x 314

3.1.2

Materials

ND

ND

3.2

Blade

3.2.1

Material

ND

ND

3.2.2

Dimensions, mm

3.2.2.1 Length

ND

160

3.2.2.2 Width

ND

80

3.2.2.3

Thickness

ND

6.5

3.2.3

Shape

ND

Rectangular

3.2.4

No. of blades

ND

15

Clearance to the shredding

3.2.5

ND

NM

cylinder, mm

3.3

Thrower

3.3.1

Material

ND

ND

3.3.2

Dimensions, mm

3.3.2.1 Length

ND

175

3.3.2.2 Width

ND

100

3.3.2.3 Thickness

ND

8.5

3.3.3

Shape

ND

Rectangular

3.3.4

Number

ND

1

Clearance to the shredding

3.3.5

ND

NM

cylinder, mm

4

Output chute

4.1

Material

4.2

Dimensions, mm

4.2.1

Length

4.2.2

Diameter/width

ND

ND

ND

ND

NM

NM

ND- No Data

Machine specifications not supplied by the manufacturer NM - Not Measured

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2018-0126 (ACT STANDARD)

Page 6 of 7

4.3

Height from the ground

ND

NM

5

Safety devices(enumerate)

ND

Belt guard

6

Special features (enumerate)

ND

None

7

Prime mover

7.1

Internal Combustion Engine

7.1.1

Brand

ND

YANMAR

7.1.2

Model

ND

TF70L

7.1.3

Make or manufacturer

ND

Indonesia

7.1.4

Serial number

ND

7.1.5

Type

ND

7.1.6

Rated power, kW

ND

ND

Four-stroke, horizontal, single cylinder diesel engine

5.22

7.1.7

Rated speed, rpm

ND

2400

7.1.8

Cooling system

ND

7.1.9

Starting system

ND

Water-cooled

Manual crank lever

7.1.10

Weight, kg

ND

NM

Machine specifications not supplied by the manufacturer

NM - Not Measured

ND- No Data

ITEMS

A. Condition of Test Materials

1. Test Material Used

2. Moisture Content (%)

3. Dimensions, L x W x T (mm)

B. Test Results

1. Speed of Components (rpm)

a. Engine

b. Shredder

2. Noise Level (dB[A])

a. Operator

b. Bagger

3. Prime Mover's (5.22 kW

YANMAR TF70L) Fuel

Consumption (L/h)

4. Input Capacity (kg/h)

RESULTS

DATA

Chopped "buko” husks

86.31

147x140 x 48

Without Load

With Load

2392

2354

966

934

92.98

93.13

1.13

1541

93.79

94.02

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2018-0126 (ACT STANDARD)

Prepared by:

Page 7 of 7

ROMULO E. EUSEBIO Test Engineer

an

MARY CHRISTINE M. AGONCILLO Test Engineer

Verified by:

DARWIN C. ARANGUREN Test Engineer

Approved for Release:

ARTHUR L. FAJARDO AMTEC Director

FEB 22 2019

Date

REMINDER

This preliminary report should not be used for advertisement or for promotion of the product

but only for evaluation and verification purposes and should not be reproduced in whole or in

part without the permission of AMTEC.

For inquiries regarding the preliminary report, you can visit us at our office, contact us at 09772047236 or 09214007137, send us a message at amtec.uplb@up.edu.ph or visit our official website at amtec.uplb.edu.ph.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.