SAVO SAMPUNG KALABAN MANK TRACTO

Machin

nery

ricultural

1977

Testing and

มองแล้ว

AMTEC

AGRICULTURAL Machinery TESTING AND Evaluation CenteR

Test Report No. 2019-0223

Agricultural Machinery - Biomass Shredder LONG FOONG WR-500

College of Engineering and Agro-Industrial Technology, University of the Philippines Los Baños, College, Laguna 4031 Email: amtec.uplb@up.edu.ph Facebook: www.facebook.com/amtec.uplbWebsite: www.amtec.uplb.edu.ph Contact Numbers: +63 921 400 7137 (Smart) / +63 977 204 7236 (Globe)

SITY

UNIVE

OF

THE

1908

PHILIPPINES

OF THE

LOS BANOS

UNIVERSITY

CEAT

PHILIPPINES

TR No. 2019-0223 (LONG FOONG WR-500)

Test Report Number

2019-0223

Page 2 of 9

Date of Issuance

Valid Until

Agricultural Machinery

Type

Brand

Model

Test Requested by

July 3, 2019

July 3, 2024

Biomass Shredder

Mobile, Cart-type, Drum Cutterhead, Horizontal Shaft

LONG FOONG

WR-500

HARBEST AGRIBUSINESS CORPORATION

No.5 Rosemarie Lane, Brgy. Kapitolyo, Pasig City, Metro Manila

SUMMARY

Performance Criteria

Requirement as per PAES 244:2010

AMTEC Test

Input capacity, kg/hr

Shredding efficiency, %, minimum

Noise level, dB(A), maximum

ND

1169.8

90

100

96

95.3

OBSERVATIONS

1. Prime Mover

2. Operator's manual (based on PAES

102:2000)

3. Previous valid test reports of

similar brand and model

4. Ease of loading

5. Number of operators

6. Transporting the machine

7. Making adjustments/Repairing

parts

8. Cleaning of parts

9. Safety

10. Other remarks

4.4 kW BRILLIANT GT600 Gasoline Engine

Not Provided

None

Loading hopper and chipping inlet are accessible to the operator

1 feeding operator, 1 assistant of the operator, and 1 optional bagger

The machine has two wheels for transport.

Belt adjustments can be easily done.

Not observed

Belt Cover

No breakdown occured

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2019 – 0223 (LONG FOONG WR-500)

Page 3 of 9

PURPOSE AND SCOPE OF TEST

Purpose

Date of Test

Location of Test

Items Tested

Performance Testing / Commercial

June 7, 2019

AMTEC, UPLB, College, Laguna

Shredding efficiency, Input capacity, Speed of components, Noise emitted by the machine, and Fuel consumption

METHODS OF TEST

The biomass shredder was tested following the Philippine Agricultural Engineering

Standards PNS/PAES 245:2010 Agricultural Machinery - Biomass Shredder- Methods of

Test.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2019-0223 (LONG FOONG WR-500)

Page 4 of 9

DESCRIPTION OF THE MACHINE

b

a

f

d

Figure 1. Parts of LONG FOONG WR-500 powered by 4.4 kW BRILLIANT gasoline engine:

a) Loading hopper, b) Chipping inlet, c) Shredding cylinder,

d) Transport wheels, e) Outlet, f) Handle.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2019 – 0223 (LONG FOONG WR-500)

SPECIFICATIONS

Page 5 of 9

Item

Manufacturer's Specifications*

AMTEC Verification

1

Main structure

1.1

Overall dimensions, mm

1.1.1

Length

ND

1420

1.1.2

Width

ND

510

1.1.3

Height

ND

1750

1.1.4 Weight without primemover, kg

ND

NM

2

Loading hopper

2.1

Material

ND

Black Iron

2.2

Bottom opening, mm

2.2.1

Length

ND

150

2.2.2

Width

ND

150

2.2.3

Height from the ground, mm

ND

940

3

Shredder assembly

3.1

Blade cover

3.1.1

Type

3.1.2

Material

ND

Drum

ND

Black Iron

3.2

Blade

3.2.1

Material

ND

Stainless Plate

3.2.2

Dimensions, mm

3.2.2.1 Length

ND

150

3.2.2.2 Width

ND

55

3.2.2.3

Thickness

ND

NM

3.2.3

Shape

ND

Rectangular

3.2.4

Number of blades

ND

2

3.2.6

Clearance to the shredding cylinder, mm

ND

9.35

4

Output chute

4.1

Material

ND

Black Iron

4.2

Dimensions, mm

4.2.1

Height

ND

175

4.2.2

Width

ND

180

4.2.3

Height from the ground

ND

525

Note: ND - No Data; NM - Not Measured

*The requesting party did not provide complete machine specifications.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2019-0223 (LONG FOONG WR-500)

SPECIFICATIONS (Cont'n)

Page 6 of 9

Item

Manufacturer's Specifications

AMTEC Verification

5

Transmission System

5.1

Type

ND

V-belt and Pulley

5.2

Engine pulley

ND

107.09 mm × 2B

5.3

Shredding cylinder pulley

ND

76.03 mm ×2B

5.4

Belt size

ND

B-33

6

Safety devices

ND

NA

7

Special features

ND

NA

8

Primemover

8.1

Brand

BRILLIANT

BRILLIANT

8.2

Model

GT600

GT600

8.3

Manufacturer

Indonesia

USA

8.4

Serial number

ND

8.5

Type

ND

8.6

Rated power, kW

4.4

8.7

Rated speed, rpm

4000

ND

Four-stroke cycle,

single cylinder,

gasoline

4.4

4000

8.8

Cooling system

Air-cooled

Air-cooled

8.9

Starting system

Rope recoil

8.10

Weight, kg

NM

Rope recoil NM

Note: ND-No Data; NM - Not Measured, NA - Not Applicable

*The requesting party did not provide complete machine specifications.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2019- 0223 (LONG FOONG WR-500)

Condition of crop

RESULTS

Page 7 of 9

Item

Data

1

Crop

Coconut Husk

2

Source

ND

3

Variety

Unknown

4

Moisture content, wet basis, %

84.66

5

Dimensions, mm

5.1

Length

5.2

Width

5.3

Thickness

139.9

92.2

29.1

ND- No Data

Performance test

Data

Item

Without Load

With Load

1

Speed of components, rpm

1.1 Engine

1.2 Shredder

3554

3355

2503

2402

2

Noise level, dB(A)

2.1 Feeding operator

92.6

95.3

2.2 Bagger (optional)

NM

NM

3

Weight, kg

NM

4

Operating time, min

7.52

5

Input capacity, kg/h

1169.8

6

Unshredded biomass material, %

0

7

Shredding efficiency, %

100

8

Fuel consumption, L/h

1.60

Note: NM-Not Measured

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2019-0223 (LONG FOONG WR-500)

Page 8 of 9

OBSERVATION

Figure 2. Quality of Output

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2019-0223 (LONG FOONG WR-500)

Suppy

LESLIE MAE O. OLIVER Test Engineer

Prepared by:

Page 9 of 9

Jashukermita

JOSHUA RANDOLPH M. GERMITA Test Engineer

ALPHONSE REGINR. MAGMANLAC

Test Engineer

Verified by:

ROMULO E. EUSEBIO

Test Engineer

Approved for Release:

Alfin to в

ARTHUR L. FAJARDO AMTEC Director

JUL 03 2019

Date

REMINDER

This test report should not be used for advertisement or for promotion of the product but only for evaluation and verification purposes and should not be reproduced in whole or in part

without the permission of AMTEC.

For inquiries regarding the test report, you can visit us at our office, contact us at 09772047236

or 09214007137, send us a message at amtec.uplb@up.edu.ph or visit our official website at

amtec.uplb.edu.ph.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.