AGRICULTURAL MACHINERY TESTING AND EVALUATION CENTER

Test Report No. 2020 – 0370

Agricultural Machinery – Mist blower STIHL SR-200

College of Engineering and Agro-Industrial Technology, University of the Philippines Los Baños, College, Laguna 4031 Email: amtec.uplb@up.edu.ph Facebook: www.facebook.com/amtec.uplb Website: www.amtec.ceat.uplb.edu.ph Contact Numbers: +63 921 400 7137 (Smart) / +63 977 204 7236 (Globe)

TR No. 2020 – 0370 (STIHL SR-200) Page 2 of 12

| Test Report Number | 2020 – 0370 |
| --- | --- |
| Date of Issuance | September 5, 2020 |
| Valid Until | September 5, 2025 |
| Agricultural Machinery | Mist blower (tested as Pesticide Sprayer) |
| Type | Backpack mist blower |
| Brand | STIHL |
| Model | SR-200 |
| Test Requested by | DEUTSCHE MOTORGERÄTE INC. 10 Tagdalit St., Brgy. Manresa, Quezon City |

SUMMARY

| Manufacturing and Performance Requirements | Requirement as per PNS/PAES 155:2010 | AMTEC Test |
| --- | --- | --- |
| Net Weight, kg, maximum | 15 | 7.89 |
| Wand | Chemical resistant polyvinylchloride or better; Length: at least 0.3 m | Plastic as observed; Length: 570 |
| Tank | Shall have a drain valve; Non-corrosive | Flexible hose attachment serves as drain valve; Non-corrosive |
| Tank Cover, Gasket, and Flexible Hose | Chemical resistant polyvinylchloride or better | Tank cover and flexible hose: Plastic as observed Gasket: Rubber |
| Hose clamp | Non-corrosive | Non-corrosive |
| Strap | Durable and non absorbent; Thickness: at least 1.5 mm; Width: at least 35 mm; Adjustable | Non-absorbent; Thickness: 1.30 mm; Width: 25 mm; Adjustable |
| Strap Pad | Shall be provided; Thickness: at least 10 mm; Width: at least 65 mm | Provided; Thickness: 11 mm; Width: 55 mm |
| Fuel Tank | Polyethylene or better; shall have provision for filtration | Plastic as observed; Provided |
| Adjustment Knob | Shall be attached to the wand | Provided |
| Cut-Off Valve | Shall be installed on the handle of the mist blower; Shall have variable setting for adjusting droplet sizes | Provided; Provided |
| Discharge Rate, L/m | 0.12-0.17 | 0.074-2.50 |
| Minimum Air Speed, m/s | 90 | 6.50a |
| Droplets Size, microns | No Data | 257.8 |

aMeasured 1 meter away from the nozzle at the lowest throttle lever setting

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College  of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2020 – 0370 (STIHL SR-200) Page 3 of 12

OBSERVATIONS

| 1. Operator’s manual (based on PAES 102:2000) |  |
| --- | --- |
| a. Language | English |
| b. Safety and warnings | Provided |
| c. Operating information | Provided |
| d. Accessories and attachments | Provided |
| e. Maintenance instruction | Provided |
| f. Storage | Provided |
| g. Handling, reception, transportation, assembly, and installation | Provided |
| h. Specifications | Provided |
| i. Dismantling and disposal | Provided |
| j. Warranty | Provided |
| k. Parts list | Provided |
| 1. Previous valid test reports of similar brand and model | None |
| 2. Manufacturer | ANDREAS STIHL AG & Co. Waiblingen, Germany |
| 3. Prime mover used | STIHL SR-200 0.8 kW Gasoline Engine |
| 4. Number of operators | One |
| 5. Presence of filtration system | Filler hole was provided with basket strainer. |
| 6. Detached welded parts | None |
| 7. Loosened bolts | Nuts and bolts were tight. |
| 8. Leakage | No leaks were observed on the tank cap during tilting test. |
| 9. Operation | The adjustment controls and cut-off valve were accessible to the operator (Figure 4). |
| 10. Safety features | The mist blower has back cushion and the straps had padding. |
| 11. Failure or abnormalities that may be observed on the machine or its component parts during and after the operation | No failure occurred during the tests. |
| 12. Other remarks | None |

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College  of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2020 – 0370 (STIHL SR-200) Page 4 of 12

PURPOSE AND SCOPE OF TEST

| Purpose | Performance Testing / Commercial |
| --- | --- |
| Dates of Test | August 25, 26 and September 2, 2020 |
| Location of Test | AMTEC Test Laboratory, UPLB, College, Laguna |
| Items Tested | Operator’s Manual, Discharge Rate, Fuel Consumption Rate, Speed of Components, Blower Range, Width of Spray, Air Velocity, Noise Level, and Number of Operators |

METHODS OF TEST

The machine was tested based on the PNS/PAES 156:2010 Agricultural Machinery – Mist Blower – Methods of Test.

DESCRIPTION OF THE MACHINE

dc

a

e

g

f

h

b

Figure 1. Parts of the STIHL SR-200 Mist blower powered by STIHL SR-200 0.8 kW  Gasoline Engine include the following: (a) Back padding, (b) Strap, (c) Flexible hose, (d) Control handle, (e) Discharge hose, (f) Wand/Blower tube,

(g) Liquid discharge metering knob, and (h) Nozzle.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College  of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2020 – 0370 (STIHL SR-200) Page 5 of 12

DESCRIPTION OF THE MACHINE (Cont’n)

a

b

e

c

d

Figure 2. Parts of the STIHL SR-200 Mist Blower powered by STIHL SR-200 0.8 kW Gasoline Engine include the following: (a) Tank cap, (b) Tank, (c) Gasoline engine, (d) Fuel tank, and (e) Rope recoil assembly.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College  of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2020 – 0370 (STIHL SR-200) Page 6 of 12

SPECIFICATIONS

| Item | Manufacturer’s Specificationa | AMTEC Verification |
| --- | --- | --- |
| 1 Overall dimensions, mm |  |  |
| 1.1 Length | ND | 340 |
| 1.2 Width | ND | 275 |
| 1.3 Height | ND | 540 |
| 1.4Weight with empty chemical and fuel tank, kg | 7.90 | 7.89b |
| 2 Tank |  |  |
| 2.1 Material | ND | Plastic |
| 2.2 Thickness, mm | ND | NM |
| 3 Tank cover |  |  |
| 3.1 Material | ND | Plastic |
| 3.2 Gasket |  |  |
| 3.2.1 Material, mm | ND | Rubber |
| 3.2.2 Thickness, mm | ND | NM |
| 4 Flexible hose |  |  |
| 4.1 Material | ND | Plastic |
| 4.2 Thickness, mm | ND | NM |
| 4.3 Length, mm | ND | 470 |
| 5 Wand |  |  |
| 5.1 Material | ND | Plastic |
| 5.2 Thickness, mm | ND | NM |
| 5.3 Length, mm | ND | 570 |
| 6 Nozzle |  |  |
| 6.1 Material | ND | Plastic |
| 6.2 Diameter, mm | ND | 60 |

aThe requesting party did not provide the complete machine specifications

bWeight includes fuel.

Note: ND – No Data, NM – Not Measured

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College  of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2020 – 0370 (STIHL SR-200) Page 7 of 12

SPECIFICATIONS (Cont’n)

| Item | Manufacturer’s Specificationa | AMTEC Verification |
| --- | --- | --- |
| 7 Engine |  |  |
| 7.1 Type | Two-stroke cycle gasoline engine | Two-stroke cycle, single vertical cylinder |
| 7.2 Brand | STIHL | STIHL |
| 7.3 Model | SR-200 | SR-200 |
| 7.4 Make/Manufacturer | ND | NM |
| 7.5 Rated power, kW | 0.8 | NM |
| 7.6 Rated speed, rpm | ND | NM |
| 7.7 Displacement, cm3 | 27.2 | NM |
| 7.8 Fuel type | Gasoline | Gasoline |
| 7.9 Fuel tank capacity, L | 1.05 | NM |
| 7.10 Cooling system | Air-cooled | Air-cooled |
| 7.11 Starting system | Rope-recoil starter | Rope-recoil starter |
| 7.12 Weight, kg | ND | NM |
| 8 Width of spray, m | ND | 0.28-1.40 |
| 9 Sprayer range, m | 9 | 2.20-8.20 |
| 10 Angle of spray, ° | ND | NM |
| 11 Special features | ND | None |

aThe requesting party did not submit complete machine specifications

Note: Note: ND – No Data, NM – Not measured

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College  of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2020 – 0370 (STIHL SR-200) Page 8 of 12

RESULTS

TESTED AS MIST BLOWER (PESTICIDE SPRAYER)

Test Condition:

Ambient Temperature, oC : 33.1

Relative Humidity, % : 62.2

| Item | Data |
| --- | --- |
| C.1 Pesticide |  |
| C.1.1 Sample Used | Water |

Sample Used for Pesticide :

Table 1. Performance of STIHL SR-200 Mist Blower at different engine throttle lever setting.

| Engine Throttle Lever Setting | Fuel Consumption Rate, L/h | Engine Speed, rpm | Noise Level, dB(A) |
| --- | --- | --- | --- |
| Low | 0.240 | 2750 | 71.1 |
| Medium | 0.300 | 4941 | 80.0 |
| High | 0.600 | 7437 | 90.7 |

Table 2. Blower air velocity at different engine throttle lever settings.

| Air Velocity, m/s |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Engine Throttle Lever Setting | Distance, m |  |  |  |  |  |  |  |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Low | 6.50 | 2.26 | 1.55 | 0.91 | 0.58 | 0.34 | 0.00 | 0.00 |
| Medium | 9.16 | 5.75 | 2.78 | 2.49 | 1.27 | 0.60 | 0.00 | 0.00 |
| High | 16.7 | 8.83 | 4.82 | 3.79 | 2.77 | 1.49 | 1.02 | 0.48 |

Table 3. Blower discharge rate at different engine throttle lever settings.

| Discharge Setting | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Discharge Rate, Lpm | 0.074 | 0.420 | 1.19 | 1.90 | 2.25 | 2.50 |

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College  of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2020 – 0370 (STIHL SR-200) Page 9 of 12

RESULTS (Cont’n)

Table 4. Blower discharge range at different discharge settings.

| Sprayer Range, m |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis charge Setting | No Baffle Screen |  |  | Deflector Screen |  |  | Conical Screen |  |  |
|  | Low | Medium | High | Low | Medium | High | Low | Medium | High |
| 1 | 2.20 | 2.80 | 3.00 | 1.30 | 1.50 | 2.60 | 1.60 | 2.60 | 1.40 |
| 2 | 2.20 | 3.50 | 3.50 | 1.60 | 1.60 | 3.20 | 1.70 | 2.60 | 3.80 |
| 3 | 2.60 | 3.90 | 3.80 | 1.60 | 3.10 | 4.00 | 1.70 | 2.90 | 4.00 |
| 4 | 3.60 | 4.00 | 4.10 | 1.80 | 3.10 | 4.40 | 1.70 | 3.00 | 5.00 |
| 5 | 3.60 | 4.00 | 6.20 | 1.70 | 3.40 | 5.40 | 1.80 | 3.20 | 5.00 |
| 6 | 3.70 | 4.30 | 8.20 | 1.90 | 4.00 | 5.40 | 1.90 | 3.60 | 4.70 |

Table 5. Blower width of spray at different discharge settings.

| Width of Spray, m |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis charge Setting | No Baffle Screen |  |  | Deflector Screen |  |  | Conical Screen |  |  |
|  | Low | Medium | High | Low | Medium | High | Low | Medium | High |
| 1 | 0.45 | 0.28 | 0.32 | 0.40 | 0.40 | 0.66 | 0.37 | 0.53 | 0.38 |
| 2 | 0.53 | 0.50 | 0.66 | 0.60 | 0.47 | 0.60 | 0.44 | 0.57 | 0.60 |
| 3 | 0.54 | 0.60 | 0.85 | 0.70 | 0.60 | 0.62 | 0.44 | 0.76 | 0.90 |
| 4 | 0.53 | 0.60 | 1.07 | 0.70 | 0.60 | 0.67 | 0.52 | 0.66 | 1.20 |
| 5 | 0.41 | 0.60 | 1.20 | 0.70 | 0.56 | 0.68 | 0.60 | 0.62 | 1.16 |
| 6 | 0.43 | 0.58 | 1.40 | 0.72 | 0.66 | 0.69 | 0.56 | 0.95 | 1.23 |

Table 6. Spray droplet size, distribution, and volume at different discharge settings.

| Spray Droplet Size, Distribution, and Volumea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Setting | 1 | 2 | 3 | 4 | 5 | 6 |
| DV 5b, μm | 252 | 274 | 238 | 272 | 296 | 215 |
| Droplet distribution, Spray Droplets/cm2 | 46.3 | 94.3 | 72.5 | 161 | 141 | 111 |
| Spray volume per unit area, μL/cm2 | 0.111 | 0.223 | 0.457 | 0.528 | 0.644 | 1.09 |
| Average DV 5b, μm | 258 |  |  |  |  |  |

aSpray droplet size, distribution and volume delivered at a distance of 4000 mm and height of 1000 mm. bDV 5 is the volume median diameter of the spray droplet.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College  of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2020 – 0370 (STIHL SR-200) Page 10 of 12

OBSERVATIONS

Figure 3. No leak was observed on tank cover during tilting test.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College  of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2020 – 0370 (STIHL SR-200) Page 11 of 12

OBSERVATIONS (Cont’n)

a

b

a

Figure 4. Operation of the STIHL SR-200 Mist Blower Mist Blower: (a) Back support and adjustable strap are provided;

(b) Adjustment control is accessible to operator.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College  of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.