achinery

Testing

and

Agrica

1977

ນາງ

aluation Co

AMTEC

AGRICULTURAL MACHINERY TESTING AND EVALUATION CENTER

OF THE

1900

Test Report No. 2023-0019

PINES

UNIVERSITY

501

BANOS

PHILIPPINES

CEAT

Agricultural Machinery - Furrower FT 4-BOTTOM FURROWER

College of Engineering and Agro-Industrial Technology, University of the Philippines Los Baños, College, Laguna 4031 Email: amtec.upib@up.edu.ph Facebook: www.facebook.com/amtec.uplb Website: www.amtec.ceat.uplb.edu.ph Contact Numbers: +63 921 400 7137 (Smart) / +63 917 704 0922 (Globe)

TR No. 2023-0019 (FT 4-BOTTOM FURROWER)

Page 2 of 9

Test Report Number

2023-0019

Date of Issuance

March 7, 2023

March 7, 2028

Valid Until

Agricultural Machinery

Type

Brand

Model

Test Requested by

Furrower

Tractor-mounted, Four shank, General-purpose bottom

FT

4- BOTTOM FURROWER

FORD TRACTOR PHILS. INC.

1384 Jose Abad Santos, Tondo, Manila

SUMMARY

Performance Criteria

Requirement as per PAES 134:2004

AMTEC Test

Depth of Furrow, mm, maximum

Width of Furrowing, mm,

ND

NDa

230 ± 17.69

2691 ± 171.56

maximum

a Not specified by the requesting party.

Note: ND-No Data

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2023-0019 (FT 4 - BOTTOM FURROWER)

OBSERVATIONS

1. Operator's manual (based on PAES 102:2000)

a. Language

b. Safety and warnings

c. Operating information

d. Accessories and attachments

e. Maintenance instruction

f. Storage

g. Handling, reception, transportation,

assembly, and installation

h. Specifications

i. Dismantling and disposal

j. Warranty

k. Parts list

2. Previous test reports of similar brand and

model

3. Manufacturer

4. Four-wheel tractor used

5. Number of operators

6. Handling and stability when machine is

working

7. Adjustments of parts

8. Straightness of furrows

9. Evenness of spacing between furrows

10. Uniformity of depth and width

11. Durability of parts (based on wear of soil- working parts, visible deformation, etc)

12. Safety features

13. Failures or abnormalities that may be observed on the machine or its components during and after the operation

14. The size of the furrower bottom shall be determined by measuring the horizontal distance from the left to the right wings of share.

15. The number of furrower bottoms shall depend on crop's row spacing and the tractor's wheel tread adjustment.

English Provided

Provided Not provided

Provided Provided

Not provided

Provided Not provided

Provided

Provided

None

Page 3 of 9

FORD TRACTOR PHILS. INC.

33.1 kW YTO MF454

One

The furrower was stable and maintained a smooth operation while it is hitched to the tractor. There were no issues encountered when maneuvering at

corners.

Depth of furrow and furrow spacing can be adjusted by moving the hitch links and standard brackets, respectively. Straightness of furrows was dependent on the operator.

675 mm ± 42.84 mm

Depth of cut: 230 mm ± 17.69 mm Width of cut: 2691 mm ± 171.56 mm No deformations were observed during the test operation.

None

None

The size of the furrower bottom is 475

mm.

The furrower has four shanks. The manufacturer's recommended spacing for corn was 70 mm.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2023-0019 (FT 4-BOTTOM FURROWER)

OBSERVATIONS (Cont❜n)

16. The height of the standards shall vary according to the crop requirement but shall not exceed 500 mm

17. The length of the toolbar shall vary according to crop's row spacing and the tractor's wheel tread adjustment.

18. The number of toolbars may be one or two depending on the manufacturer's design. 19. Cast iron and/or mild steel shall be used in the manufacture of the moldboard, standard and toolbar.

20. Carbon steel with at least 80% carbon content (e.g. AISI 1080) or alloy steel with at least 0.0005% boron content shall be used in the manufacture of share.

21. The hitch of the furrower shall be

compatible with the three-point linkage of the four-wheel tractor as specified in PAES 118.

22. The furrower shall be easy to hitch to and unhitch from the tractor as well as adjust the spacing between furrows.

23. The furrower shall be

be free from manufacturing defects such as sharp edges and surfaces that may be detrimental to the operator.

24. Except for furrower bottoms, other uncoated metallic surfaces shall be free from rust and shall be painted properly.

25. Other remarks

Page 4 of 9

The height of the standards is 450 mm

The length of the tool bar is 2450 mm.

The furrower has one tool bar.

Steel was used for the manufacture of moldboard, standard, and toolbar.

The material used for the share was not verified.

Not verified

Hitching and unhitching were easy. Adjusting the furrower spacing can be done by loosening the bolts and nuts used to attach the shanks to the toolbars.

The furrower was free from manufacturing defects during the test operation.

All parts were painted except for the furrower bottom.

No markings or labelling provided on the furrower.

Purpose

Date of Test

Location of Test

Items Tested

PURPOSE AND SCOPE OF TEST

Performance Testing / Commercial

January 20, 2023

Luisita Access Road, Tarlac City, Tarlac

15°26'5.93" N, 120°39'48.13" E

Operator's Manual, Depth of Furrow, Width of Furrow, Field Capacity, Field Efficiency, Noise Level, and Fuel Consumption

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2023-0019 (FT 4 - BOTTOM FURROWER)

Page 5 of 9

METHODS OF TEST

The furrower was tested based on the PAES 135:2004 - Agricultural Machinery - Furrower - Methods of Test.

DESCRIPTION OF THE MACHINE

b

a

Figure 1. Parts of the FT 4 - BOTTOM FURROWER Furrower hitched to 33.1 kW YTO MF454 include the following: (a) Toolbar, (b) Standard,

(c) Moldboard, and (d) Upper-link hitch point.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2023-0019 (FT 4-BOTTOM FURROWER)

SPECIFICATIONS

Page 6 of 9

Item

Manufacturer's

Specification

AMTEC Verification

1

Overall dimensions and weight

1.1

Length, mm

2530

770

1.2

Width, mm

850

2525

1.3

Height, mm

1240

1100

1.4

Weight, kg

ND

NM

2

Furrower bottom

2.1

Quantity

4

4

2.2

Size, L × W, mm

ND

2450 × 85

2.3

Material

Leafspring

ND

3

Standard

3.1

Dimension, H× W × t, mm

3.2

Material

600 × 250 × 30 MS plate

450 x 210 x 24.91

ND

4

Toolbar

4.1

Dimension, L × t, mm

2210 × 127

2450 × 85

4.2

Material

Mild steel angle bar

ND

5

Features

5.1

Recommended furrow spacing, mm

ND

700b

5.2

Type of crops

ND

Cornb

6

Operating width, mm

ND

2525

a The data were obtained from the operator's manual and specifications sheet provided by the requesting party. b Specified by the requesting party during field test

Note: ND-No Data, NM - Not Measured

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2023-0019 (FT 4-BOTTOM FURROWER)

RESULTS

Page 7 of 9

Item

Data

1

Test Conditions

1.1

Dimensions of field, m

1.1.1

Length

1.1.2

Width

1.2

Area, m2

46.0

23.0

1058.00

1.3

Soil type

1.4

Soil moisture content, %

1.5

Last crop planted

1.6

Field condition before operation

2

Field Performance

2.1

Type of field operation

2.2

Tractor's gearshift setting

2.3

Traveling speed, kph

2.4

2.5

2.6

Average depth of furrow, mm

Average width of furrowing, mm

Actual field capacity

Sandy loam

9.98 Mungbean Harrowed

Secondary tillage - Furrowing

4th

gear - Low

5.52

230 ± 17.69

2691 ± 171.56

2.6.1

ha/h

1.18

2.6.2

h/ha

0.853

2.7

Theoretical field capacity

2.7.1

ha/h

1.40

2.7.2

h/ha

0.721

2.8

Field efficiency, %

84.66

2.9

Noise at operator's ear level, dB(A)

91.0

2.10

Wheel slip, %

NM

2.11

Fuel consumption

2.11.1 L/h

8.30

2.11.2

L/ha

7.09

2.13

Method of operation

Continuous pattern turn strips at each end

Note: NM-Not Measured

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2023-0019 (FT 4 - BOTTOM FURROWER)

Page 8 of 9

OBSERVATION

Figure 2. Quality of work

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.

TR No. 2023-0019 (FT 4 - BOTTOM FURROWER)

Tested by:

Page 9 of 9

ALBALOS

RHEY MARC C. ALBALOS Test Engineer

CHRISTOPHER S. JAPONES Test Engineer

REMINDER

Verified by:

Ma

MARIE JEHOSA B. REYES Test Engineer

Approved for Release:

Alfine

вер

ARTHUR L. FAJARDO AMTEC Director

MAR 10 2023

Date

This test report should not be used for advertisement or for promotion of the product but only for evaluation and verification purposes and should not be reproduced in whole or in part without the permission of AMTEC.

For inquiries regarding the test report, you can visit us at our office, contact us at +63 921 400

7137 or +63 917 704 0922, send us a message at amtec.uplb@up.edu.ph or visit our official

website at www.amtec.ceat.uplb.edu.ph.

This is an official document of the Agricultural Machinery Testing and Evaluation Center (AMTEC) of the College of Engineering and Agro-Industrial Technology (CEAT) University of the Philippines Los Baños (UPLB) Falsification of document is punishable by law under Article 172 of the Revised Penal Code.